## AMBA (Advanced Microcontroller Bus Architecture)

### What is AMBA?

AMBA is a standardized bus architecture developed by ARM to facilitate communication between the processor, memory, and peripherals in System-on-Chip (SoC) designs.

Goal: Ensure high performance, modularity, and easy IP (Intellectual Property) integration across ARM-based systems.

### AMBA Bus Types (Versions)

| Bus | Purpose |
| --- | --- |
| ASB (Advanced System Bus) | Older synchronous bus |
| AHB (Advanced High-performance Bus) | High-speed, burst transfer, pipelined bus |
| APB (Advanced Peripheral Bus) | Low-speed, simple for peripherals |
| AXI (Advanced eXtensible Interface) | High-performance, for modern SoCs |
| ACE (AXI Coherency Extensions) | For cache-coherent multiprocessor systems |

## AHB (Advanced High-performance Bus)

### What is AHB?

AHB is a high-speed, pipelined bus protocol under the AMBA specification, typically used for high-performance peripherals and memory communication.

It was introduced in AMBA 2.0.

### Key Features of AHB:

- Single Clock Edge Operation

- Pipelined Architecture → High throughput

- Burst Transfers → Sequential transfers in one request

- Single Master, Multiple Slave (standard)

- Split/Retry Response → Improves bus efficiency

- Supports 32-bit, 64-bit, and 128-bit data buses

### AHB Bus Signals (Basic):

| Signal | Description |
| --- | --- |
| HADDR | Address bus |
| HWDATA | Write data bus |
| HRDATA | Read data bus |
| HWRITE | Write control signal |
| HTRANS | Transfer type |
| HREADY | Transfer completion |
| HRESP | Transfer response |

### AHB vs APB

| Feature | AHB | APB |
| --- | --- | --- |
| Speed | High | Low |
| Complexity | Complex | Simple |
| Use | Memory, DMA, high-speed devices | UART, GPIO, Timer |
| Burst Support | Yes | No |
| Pipelining | Yes | No |