## Install Keil µVision (IDE for ARM7)

### 🔹 Step 1: Download

- Visit https://www.keil.com/download/

- Download Keil MDK-ARM (µVision IDE).

### 🔹 Step 2: Install Keil

- Run the installer.

- Follow on-screen instructions and complete the installation.

### 🔹 Step 3: Add Legacy Support for LPC2148

- Visit: https://www.keil.com/dd2/legacy/

- Download the Legacy Device Support Pack (for NXP LPC2148).

- Install the pack by running the .exe file.

## 🧰 2. Install Proteus (for Simulation)

### 🔹 Step 1: Download

- Go to https://www.labcenter.com or use an offline setup.

- Download Proteus 8 Professional or Proteus Design Suite.

### 🔹 Step 2: Install Proteus

- Run the setup and install.

- Activate if needed.

### 🔹 Step 3: Add LPC2148 Library (if not available)

- Download LPC2148 Proteus model files (.LIB and .IDX).

- Paste into Proteus/LIBRARY folder.

- Restart Proteus.

## 💻 3. Create LPC2148 Project in Keil

### 🔹 Step 1: Open Keil µVision

- Project → New µVision Project

- Name your project and save.

### 🔹 Step 2: Select Device

- Select NXP (Philips) → LPC2148.

### 🔹 Step 3: Startup Code

- Add startup file (use default if prompted).

### 🔹 Step 4: Add Source Code

- Create a new .c file.

- Example: LED Blinking code.

- Add file to “Source Group 1”.

### 🔹 Step 5: Configure Target

- Project → Options for Target

  - Enable Create HEX File

  - Set clock frequency (usually 12 MHz or as per your circuit)

### 🔹 Step 6: Build

- Press F7 to compile.

- HEX file will be generated in the project folder.

## 🔌 4. Simulate LPC2148 in Proteus

### 🔹 Step 1: Open Proteus

- Create a new project or schematic.

### 🔹 Step 2: Add Components

- Add LPC2148 (ARM7 microcontroller).

- Add LEDs, resistors, crystal oscillator (12 MHz), capacitors, etc.

### 🔹 Step 3: Load HEX File

- Double-click LPC2148 → set the Program File to your HEX file from Keil.

### 🔹 Step 4: Run Simulation

- Click the play button.

- Your embedded C code will run in Proteus!

## 🔁 Optional: Load to Real Hardware (via Flash Magic)

### 🔹 Step 1: Install Flash Magic

- Download from: https://www.flashmagictool.com

### 🔹 Step 2: Connect LPC2148 Board

- Use USB/Serial interface and connect to PC.

### 🔹 Step 3: Upload HEX

- Select LPC2148 device, COM port, and load HEX file.