## Introduction to Keil, Proteus, and Basic Program with LPC2148

### ✅ 1. Introduction to Keil µVision

Keil µVision is an Integrated Development Environment (IDE) used for writing, compiling, and debugging embedded C programs, especially for ARM-based microcontrollers.

#### 🔹 Features:

- Supports ARM7, Cortex-M, 8051, and more.

- Inbuilt compiler and debugger.

- User-friendly interface.

### ✅ 2. Introduction to Proteus

Proteus Design Suite is a simulation software used for designing and testing circuits without physical hardware.

#### 🔹 Features:

- Simulates microcontrollers and embedded systems.

- Visual testing of circuits using virtual components.

- Allows HEX file loading for real-time program simulation.

### ✅ 3. Stepwise Process: Keil + Proteus + LPC2148

### 🧾 Step 1: Install Keil µVision and Proteus

- Download and install Keil from https://www.keil.com.

- Download and install Proteus from https://www.labcenter.com.

### 🧾 Step 2: Create New Project in Keil

- Open Keil → Project → New µVision Project

- Name and save your project.

- Select Device: NXP (Philips) → LPC2148.

- Add startup code when prompted.

### 🧾 Step 3: Write Basic Program (LED Blinking)

c

CopyEdit

#include <lpc214x.h>

void delay() {

for (int i = 0; i < 100000; i++);  // Rough delay loop

}

int main() {

IO0DIR = 0x000000FF; // Set P0.0 to P0.7 as output

while(1) {

IO0SET = 0x000000FF; // Turn ON LEDs

delay();

IO0CLR = 0x000000FF; // Turn OFF LEDs

delay();

}

}

### 🧾 Step 4: Build the Project

- Right-click Source Group → Add .c file.

- Click Build (F7) → Check for errors.

- Ensure Create HEX File is enabled in Project → Options for Target.

### 🧾 Step 5: Simulate in Proteus

- Open Proteus → New Project → Design Schematic.

- Add LPC2148, LEDs, resistors, oscillator (12 MHz), etc.

- Double-click LPC2148 → Load HEX file from Keil.

- Click Run → Watch LEDs blink as per your program.

### 🧾 Step 6: Optional - Upload to Hardware

- Use Flash Magic software to load the HEX file to a physical LPC2148 development board.