Demonstration Experiment on Gas Chromatography (GC)

Demonstration Experiment: Separation and Identification of Volatile Compounds by Gas Chromatography

Objective:

To demonstrate the separation of a mixture of volatile organic compounds using gas chromatography and to interpret chromatographic data (retention time and peak area).

Principle:

Gas Chromatography is a separation technique where the components of a volatile sample are distributed between a mobile gas phase (carrier gas) and a stationary phase (usually a liquid coated on a solid support inside a column). Components are separated based on differences in volatility and interaction with the stationary phase.

Apparatus & Materials:

- Gas chromatograph with thermal conductivity detector (TCD) or flame ionization detector (FID)

- Capillary or packed GC column (e.g., non-polar column like DB-1 or polar like Carbowax)

- Syringes (1–10 µL)

- Computer with chromatography software

- Carrier gas (usually Helium or Nitrogen)

- Sample mixture: e.g., mixture of ethanol, acetone, and toluene

Chemicals:

- Ethanol (C₂H₅OH)

- Acetone (CH₃COCH₃)

- Toluene (C₆H₅CH₃)

- Methanol or other volatile solvents (optional)

Experimental Procedure:

- Instrument Setup:

  - Turn on the GC system and allow it to equilibrate (column temperature, detector, and injector).

  - Set the oven temperature: e.g., start at 40°C and ramp up to 200°C at 10°C/min.

  - Set detector and injector temperatures (usually ~250°C).

  - Ensure carrier gas flow is stable (e.g., 1 mL/min for He).

- Sample Preparation:

  - Prepare a diluted solution of ethanol, acetone, and toluene in methanol or water.

  - Load 1–2 µL of the mixture into a clean syringe.

- Injection:

  - Inject the sample into the GC injector port quickly and smoothly.

  - Start the chromatographic run and record the chromatogram.

- Observation:

  - Monitor the chromatogram for peaks corresponding to each component.

  - Record retention times and peak areas.

Results and Interpretation:

| Compound | Expected Retention Time (approx.) | Peak Area | Remarks |
| --- | --- | --- | --- |
| Acetone | ~2.5 min | X | Most volatile |
| Ethanol | ~3.5 min | X | Moderate volatility |
| Toluene | ~6.0 min | X | Least volatile |

- Peaks are identified based on known standards.

- The area under each peak is proportional to the amount of compound.

Conclusion:

This demonstration illustrates the principle of gas chromatography, showing how volatile compounds are separated based on their interactions with the column and their boiling points. The retention time is a key parameter for qualitative analysis, and peak area is used for quantification.

Precautions:

- Ensure all fittings are leak-tight.

- Use high-purity carrier gas.

- Do not overload the column with too much sample.

- Handle volatile organic compounds in a fume hood.